PROJECT REPORT DRAFT

Design and Implementation of a Leakage-Aware Radar Target Classification System with a Custom Streaming CNN Accelerator on AMD Kria KV260

## TITLE PAGE

**Project Title:**

**Design and Implementation of a Leakage-Aware Radar Target Classification System with a Custom Streaming CNN Accelerator on AMD Kria KV260**

**Submitted by:**

[Student Name]

[Register Number / Roll Number]

**Submitted to:**

[Guide Name]

[Department]

[Institution]

**Internship / Academic Project Organization:**

[Organization Name]

**Academic Year:**

[Year]

**Project Duration:**

[Start Date] to [End Date]

---

## CERTIFICATE

This is to certify that the project titled **“Design and Implementation of a Leakage-Aware Radar Target Classification System with a Custom Streaming CNN Accelerator on AMD Kria KV260”** has been carried out by **[Student Name]** under the guidance of **[Guide Name]** during the period **[Project Period]**. The work presented in this report is submitted in partial fulfilment of the requirements of **[Degree / Internship / Course]**.

**Guide Signature:** ____________________

**Head of Department:** ____________________

**External Examiner:** ____________________

**Date:** ____________________

---

## DECLARATION

I hereby declare that the work presented in this report is my original contribution, except where references have been explicitly provided. The project was carried out under the guidance of **[Guide Name]**. The results, software, hardware architecture, and analysis presented in this document have not been submitted elsewhere for any other academic award.

**Student Signature:** ____________________

**Date:** ____________________

---

## ACKNOWLEDGEMENT

I express my sincere gratitude to **[Guide Name]**, **[Department / Organization]**, and all faculty members and technical staff who supported this project. I also acknowledge the developers of the software tools, frameworks, datasets, and documentation used during the work. Special thanks are extended to **[Names / Teams]** for guidance in radar signal processing, machine learning validation, Vitis HLS, and Vivado system integration.

---

# ABSTRACT

This project presents an end-to-end radar target classification pipeline intended for deployment on the AMD Kria KV260 platform. The system transforms complex radar in-phase and quadrature data into normalized 100×100 micro-Doppler spectrograms and classifies each observation into one of four categories: Drone, Bird, Human, and Corner Reflector.

The project began with a software-based 2D convolutional neural network and a conventional window-level train/test split. Although the initial model produced high accuracy, further investigation identified a risk of optimistic evaluation because overlapping windows from the same source recording could appear in different dataset partitions. A leakage-aware record-wise preprocessing pipeline was therefore implemented. The improved validation method keeps all windows derived from a single source recording in exactly one split. A grouped five-fold evaluation was subsequently introduced to provide a more defensible estimate of generalization.

A compact depthwise-separable CNN was selected for hardware implementation. Two deployment routes were explored. The first used the AMD Vitis AI DPU flow through PyTorch, ONNX export, quantization, XModel generation, and KV260 compilation. The second implemented the CNN as a fully custom Vitis HLS accelerator. The initial frame-buffered HLS design was functionally correct but exceeded the available on-chip memory resources because complete intermediate feature maps were stored between layers. The design was therefore rewritten using a streaming DATAFLOW architecture with line buffers and FIFO-based inter-stage communication.

The streaming accelerator passed C simulation, HLS synthesis, RTL co-simulation, Vivado IP packaging, block-design integration, synthesis, implementation, and timing closure on the K26 device. Post-implementation utilization was approximately 83.1% LUT, 30.6% flip-flop, 54.6% DSP, 17.7% block RAM, and 0% UltraRAM. Final timing closure achieved positive worst slack. Physical KV260 execution, measured end-to-end latency, power consumption, and CPU/DPU/HLS benchmarking remain to be completed and are intentionally left as future validation items in this report.

**Keywords:** Radar classification, micro-Doppler, 2D CNN, depthwise-separable convolution, FPGA, Kria KV260, Vitis HLS, streaming architecture, DATAFLOW, record-wise validation.

---

# TABLE OF CONTENTS

| Chapter | Title | Suggested Page |

|---|---|---:|

| 1 | Introduction | 1 |

| 2 | Problem Statement, Objectives, and Scope | 6 |

| 3 | Background and Literature Review | 11 |

| 4 | Proposed System and Project Evolution | 19 |

| 5 | Requirements, Constraints, and Assumptions | 27 |

| 6 | Dataset Understanding and Signal Preprocessing | 34 |

| 7 | Machine Learning Architecture and Training | 45 |

| 8 | Validation Methodology and Generalization Analysis | 56 |

| 9 | Vitis AI DPU Deployment Path | 68 |

| 10 | Custom HLS Accelerator Architecture | 78 |

| 11 | Streaming Redesign and Hardware Optimization | 91 |

| 12 | Vivado System Integration | 104 |

| 13 | Verification and Testing | 115 |

| 14 | Results and Discussion | 128 |

| 15 | Comparative Analysis and Ablation Studies | 142 |

| 16 | Limitations, Applications, and Future Work | 154 |

| 17 | Conclusion | 163 |

| 18 | References | 167 |

| Appendix | Supporting Evidence and User Procedure | 172 |

> Page numbers must be regenerated after final formatting.

---

CHAPTER 1

INTRODUCTION

1.1 Radar Target Classification

Radar systems transmit electromagnetic energy and analyze reflected signals to detect objects and characterize their motion. Conventional detection can determine the presence, range, and radial motion of a target, but distinguishing between objects with similar radar signatures remains difficult. A small drone, a bird, and a moving human can produce overlapping characteristics under some observation conditions.

Micro-Doppler analysis provides additional information by capturing small motion components caused by propellers, wings, limbs, and body movement. When represented as a time-frequency spectrogram, these motion patterns can be processed using image-based machine-learning methods.

1.2 Motivation for 2D CNN Processing

A spectrogram contains structured information along both time and frequency axes. A 2D convolutional neural network can learn local patterns such as harmonic structures, periodic motion, transient energy, and time-varying Doppler components. The project therefore converts radar IQ segments into 100×100 spectrogram images and applies a compact 2D CNN.

The 2D approach was selected to preserve more time-frequency structure than a single one-dimensional feature vector while remaining small enough for edge deployment.

1.3 Need for FPGA Acceleration

CNN inference consists mainly of repeated multiply-accumulate operations. General-purpose processors execute these operations sequentially or with limited parallelism. FPGA devices allow parallel arithmetic, customized memory access, fixed-point computation, and deterministic pipelines.

The Kria KV260 was selected because it combines an Arm processing system with programmable logic. This supports a heterogeneous architecture in which preprocessing and control can execute on the processing system while CNN inference executes in custom hardware.

1.4 Scientific Validation Problem

The first successful model used overlapping windows. When windows from the same source recording are randomly divided between training and testing, the test data can be very similar to the training data. This does not necessarily mean labels are directly leaked, but it can produce an optimistic performance estimate.

The project therefore distinguishes between:

1. Window-level engineering evaluation, useful for proving the pipeline works.

2. Recording-wise evaluation, where complete source recordings remain disjoint.

3. Grouped k-fold evaluation, which measures variation across several held-out recording groups.

This distinction is central to the report and should not be hidden.

1.5 Project Contributions

The main contributions are:

- A real radar IQ inspection and conversion pipeline.

- A 100×100 spectrogram preprocessing flow.

- A four-class depthwise-separable CNN.

- Leakage-aware record-wise partitioning.

- Grouped five-fold validation.

- A Vitis AI DPU-compatible model path.

- A custom fixed-point HLS implementation.

- A frame-buffered-to-streaming hardware redesign.

- RTL co-simulation and post-implementation timing closure on the K26 target.

- A reproducible evidence trail linking software inference, fixed-point inference, RTL verification, and Vivado implementation.

1.6 Organization of the Report

Chapter 2 defines the problem, objectives, scope, and success criteria. Chapter 3 discusses radar micro-Doppler classification and FPGA deployment approaches. Chapter 4 explains the proposed system and the project’s evolution. Chapters 5 through 8 cover requirements, preprocessing, model training, and validation. Chapters 9 through 12 describe DPU deployment, custom HLS architecture, streaming optimization, and Vivado integration. Chapters 13 through 15 present verification, results, and comparisons. Chapter 16 discusses limitations and future work, followed by the conclusion and appendices.

---

CHAPTER 2

PROBLEM STATEMENT, OBJECTIVES, AND SCOPE

2.1 Problem Statement

The project addresses three connected problems:

Classification problem: radar targets with different physical behaviour must be separated using time-frequency features.

Validation problem: overlapping windows can create an optimistic test result when windows from the same recording appear in multiple partitions.

Deployment problem: a trained CNN must be converted into a resource-efficient hardware architecture that fits the KV260 and meets timing.

2.2 Primary Objective

To design and validate a four-class radar target classification system using 100×100 spectrograms and deploy its CNN inference stage as a custom fixed-point streaming hardware accelerator on the Kria KV260 programmable logic.

2.3 Secondary Objectives

- Inspect and understand the supplied `radar_iq.npy` format.

- Convert eligible source recordings into fixed-length IQ windows.

- Generate normalized spectrogram images.

- Train a compact CNN for Drone, Bird, Human, and Corner Reflector classification.

- Prevent source-recording overlap between training and test partitions.

- Compare window-level and recording-level performance.

- Export and verify the model through ONNX.

- Complete an alternative Vitis AI DPU compilation path.

- Implement HLS C simulation and RTL co-simulation.

- Reduce on-chip memory through line buffers, streams, and DATAFLOW.

- Integrate the accelerator with the Zynq UltraScale+ processing system.

- Close implementation timing on the K26 device.

- Prepare for physical-board inference and benchmarking.

2.4 Present Scope

Completed scope includes:

- Dataset inspection.

- IQ window generation.

- Spectrogram generation.

- Model training and evaluation.

- Grouped recording-wise validation.

- Vitis AI compilation.

- Fixed-point HLS implementation.

- Streaming accelerator redesign.

- C simulation.

- HLS synthesis.

- RTL co-simulation.

- Vivado IP packaging and block-design integration.

- Post-implementation resource analysis.

- Timing closure.

2.5 Items Not Yet Completed

The following are deliberately left open:

| Pending Item | Evidence to Insert Later |

|---|---|

| Physical KV260 execution | [Insert board photograph and console output] |

| Bitstream/XSA archival verification | [Insert final file paths and checksums] |

| Bare-metal or Linux host software | [Insert application architecture] |

| Measured accelerator-only latency | [Insert mean, minimum, maximum, standard deviation] |

| End-to-end latency including preprocessing | [Insert measured value] |

| CPU baseline on KV260 Arm cores | [Insert measured value] |

| Vitis AI DPU board benchmark | [Insert measured value] |

| HLS vs DPU speedup | [Insert result] |

| Board power measurement | [Insert idle and active power] |

| Full-dataset hardware accuracy | [Insert confusion matrix] |

| Live radar input | [Describe interface or mark not implemented] |

2.6 Success Criteria

The project shall be considered functionally successful when:

- The software model produces repeatable predictions.

- Source recording overlap is zero in strict evaluation.

- Fixed-point outputs are sufficiently close to the software reference.

- C simulation passes.

- RTL co-simulation passes.

- The custom accelerator fits the K26 target.

- Vivado implementation meets timing.

- Physical hardware executes at least one validated test sample. **[Pending]**

- Hardware outputs match the fixed-point reference. **[Pending]**

CHAPTER 3

BACKGROUND AND LITERATURE REVIEW

3.1 Micro-Doppler Signatures

Describe how rotating blades, wing motion, walking limbs, and rigid reflectors produce different time-frequency patterns. Include one representative spectrogram for each class.

**Insert Figure 3.1:** Drone spectrogram

**Insert Figure 3.2:** Bird spectrogram

**Insert Figure 3.3:** Human spectrogram

**Insert Figure 3.4:** Corner Reflector spectrogram

3.2 Spectrogram-Based Classification

Explain short-time Fourier transformation, magnitude conversion, logarithmic scaling, normalization, resizing, and the use of CNNs for learning local time-frequency patterns.

3.3 CNNs for Edge Classification

Discuss the difference between standard convolution and depthwise-separable convolution. A depthwise-separable block separates spatial filtering from channel mixing:

1. Depthwise 3×3 convolution processes each channel independently.

2. Pointwise 1×1 convolution combines information across channels.

This reduces parameter count and is well suited to embedded inference.

3.4 CPU, DPU, and Custom FPGA Approaches

CPU

Flexible, simple to program, but limited by instruction execution and memory hierarchy.

Vendor DPU

Provides a supported deployment route for compatible neural-network operators. It reduces development time but relies on a fixed accelerator architecture and supported operator set.

Custom HLS Accelerator

Allows the datapath, fixed-point width, memory architecture, parallelism, and interfaces to be tailored to one model. Development and verification effort are higher, but the design offers architectural control.

3.5 Existing System Comparison

| Approach | Main Strength | Main Limitation | Relevance |

|---|---|---|---|

| CPU-only CNN | Easy development | Lower deterministic throughput | Software baseline |

| Vitis AI DPU | Fast deployment | Vendor architecture and operator constraints | Reference deployment |

| Frame-buffered HLS | Simple C++ mapping | Excessive intermediate memory | Functional baseline |

| Streaming HLS | Low feature-map storage | More complex design | Final custom accelerator |

## 3.6 Literature Review Table

Replace placeholders with verified sources.

| Ref. | Work | Input Representation | Model | Target Hardware | Validation Split | Key Result | Gap Addressed Here |

|---|---|---|---|---|---|---|---|

| [R1] | [Paper title] | [1D/2D] | [Model] | [Hardware] | [Split] | [Metric] | Recording-wise validation |

| [R2] | [Paper title] | Spectrogram | CNN | FPGA | [Split] | [Metric] | Custom streaming architecture |

| [R3] | [Paper title] | Radar IQ | [Model] | CPU/GPU | [Split] | [Metric] | Edge deployment |

| [R4] | Original reference repository | [Describe] | [Describe] | [Describe] | [Describe] | [Describe] | 100×100 2D CNN and KV260 |

> Do not enter paper accuracy values until the exact paper, dataset protocol, and validation split have been verified.

---

CHAPTER 4

PROPOSED SYSTEM AND PROJECT EVOLUTION

4.1 Final System Concept

Raw radar IQ recording

↓

Recording-wise eligibility check

↓

Fixed-length IQ window generation

↓

Top-k range-bin energy selection

↓

100×100 normalized spectrogram

↓

Depthwise-separable 2D CNN

↓

Four output logits

↓

Predicted target class

The uploaded chapter-wise template has a useful academic flow—front matter, introduction, objectives, architecture, implementation, testing, results, limitations, conclusion, references, and appendices. I have adapted that structure to your radar-classification and custom-FPGA-accelerator project.

Recommended Project Title

Design and Implementation of a Leakage-Aware Radar Target Classification System with a Custom Streaming CNN Accelerator on AMD Kria KV260

Alternative shorter title:

FPGA-Accelerated Radar Target Classification Using a Streaming 2D CNN on Kria KV260

ABSTRACT

This project presents an end-to-end radar target classification system designed for deployment on the AMD Kria KV260 platform. Complex radar in-phase and quadrature data are converted into normalized 100×100 micro-Doppler spectrograms and classified into four target categories: Drone, Bird, Human, and Corner Reflector.

The work began with a software-based two-dimensional convolutional neural network and an overlapping window-level dataset split. Although this initial approach achieved high classification accuracy, further analysis identified a risk of optimistic validation because windows originating from the same radar recording could appear in both training and testing partitions. A leakage-aware record-wise preprocessing method was therefore implemented. Under this method, source recordings are divided before window generation so that all windows from one recording remain in exactly one dataset partition.

The selected machine-learning model is a compact depthwise-separable convolutional neural network with approximately 24,612 trainable parameters. The final training approach uses top-three energy range-bin averaging, train-only augmentation, capped class weights, and grouped five-fold recording-disjoint validation. The grouped five-fold evaluation achieved a window-level accuracy of 80.57% ± 2.98%, a macro F1-score of 66.45% ± 5.29%, and a recording-level accuracy of 86.67% ± 11.18%, while maintaining zero source-recording overlap.

Two hardware deployment approaches were investigated. The first used the AMD Vitis AI DPU flow, including PyTorch training, ONNX export, quantization, XModel generation, and KV260 compilation. The second implemented the complete CNN as a custom fixed-point accelerator using Vitis HLS. The initial HLS architecture stored complete intermediate feature maps and exceeded the available BRAM and UltraRAM resources. The accelerator was subsequently redesigned as a streaming DATAFLOW architecture using FIFO channels, line buffers, channel-vector processing, and depthwise-separable convolution stages.

The final streaming accelerator successfully completed C simulation, HLS synthesis, RTL co-simulation, Vivado IP packaging, Zynq processing-system integration, synthesis, implementation, and timing closure. Post-implementation utilization was approximately 83.1% LUT, 30.6% flip-flop, 54.6% DSP, 17.7% BRAM, and 0% UltraRAM. Final timing analysis reported positive worst setup slack. Physical KV260 execution, measured latency, throughput, power consumption, and CPU/DPU/HLS performance comparison remain to be completed.

Keywords: Radar classification, micro-Doppler, 2D CNN, depthwise-separable CNN, FPGA, Kria KV260, Vitis HLS, streaming architecture, DATAFLOW, record-wise validation.

Recommended Chapter Structure

CHAPTER 1 — INTRODUCTION

1.1 Radar Target Classification

Explain that radar systems can detect range and velocity, but distinguishing targets such as drones, birds, and humans requires analysis of more detailed motion characteristics.

1.2 Micro-Doppler Signatures

Explain how:

Drone propellers generate periodic blade-related components.

Bird wings generate flapping patterns.

Human motion produces torso and limb components.

Corner reflectors generally produce rigid and stable reflections.

Add one real spectrogram image for every class.

1.3 Why Spectrograms Are Used

A spectrogram converts time-domain radar data into a time-frequency image. This allows a 2D CNN to detect spatial patterns, harmonics, periodic motion, and Doppler variation.

1.4 Need for Edge Deployment

Explain why radar inference may need:

Low latency

Deterministic execution

Reduced dependence on a desktop GPU

Real-time embedded processing

Lower communication bandwidth

1.5 Why the KV260 Was Selected

Discuss:

Integrated Arm processing system

Programmable logic

DSP resources

BRAM and UltraRAM

AXI interfaces

Support for both DPU and custom HLS deployment

1.6 Project Overview

Use this flow:

Raw Radar IQ

↓

Window Generation

↓

Range-Bin Selection

↓

100×100 Spectrogram

↓

Depthwise-Separable CNN

↓

Drone / Bird / Human / Corner Reflector

1.7 Major Contributions

State the actual contributions:

Real radar dataset inspection.

Leakage-aware record-wise preprocessing.

Grouped five-fold validation.

Compact hardware-oriented DS-CNN.

Vitis AI DPU compilation.

Custom fixed-point HLS implementation.

Streaming line-buffered architecture.

Vivado integration and timing closure.

CHAPTER 2 — PROBLEM STATEMENT, OBJECTIVES, AND SCOPE

2.1 Problem Statement

The project addresses three problems.

Classification problem

Radar targets may have similar energy or velocity characteristics, making them difficult to distinguish using simple thresholds.

Validation problem

Overlapping windows from the same recording can appear in both training and testing data, producing an optimistic accuracy value.

Hardware problem

A CNN that works in software may consume excessive memory or arithmetic resources when directly translated into FPGA hardware.

2.2 Primary Objective

To design a four-class radar target classifier using 100×100 spectrograms and implement its inference stage as a custom fixed-point streaming accelerator on the Kria KV260.

2.3 Secondary Objectives

Understand the original .npy dataset.

Extract fixed-length radar windows.

Generate spectrogram images.

Train and evaluate a compact CNN.

Prevent source-recording leakage.

Compare window-level and record-level metrics.

Export the model through ONNX.

Compile a Vitis AI DPU version.

Design an HLS accelerator.

Reduce BRAM usage using streaming architecture.

Complete RTL co-simulation.

Integrate the IP with the Zynq PS.

Meet implementation timing.

2.4 Completed Scope

Activity	Status

Dataset inspection	Complete

Window generation	Complete

Spectrogram generation	Complete

CNN training	Complete

Record-wise validation	Complete

Grouped five-fold validation	Complete

ONNX export	Complete

Vitis AI quantization	Complete

DPU compilation	Complete

HLS C simulation	Complete

HLS synthesis	Complete

RTL co-simulation	Complete

Streaming redesign	Complete

Vivado integration	Complete

Timing closure	Complete

2.5 Work Not Yet Completed

Leave this table in the final report.

Activity	Status	Evidence to Insert

Physical KV260 programming	[Pending/Confirm]	[Bitstream programming screenshot]

XSA export	[Pending/Confirm]	[Final XSA path]

Host application	Pending	[Source-code path]

Single-sample board inference	Pending	[Console output]

Full test-set hardware inference	Pending	[Confusion matrix]

HLS accelerator latency	Pending	[Mean/minimum/maximum]

End-to-end latency	Pending	[Preprocessing + transfer + inference]

CPU benchmark	Pending	[Latency/FPS]

DPU board benchmark	Pending	[Latency/FPS]

FPGA power measurement	Pending	[Idle/active power]

Live radar input	Pending	[Hardware interface description]

CHAPTER 3 — LITERATURE REVIEW AND EXISTING SYSTEM

3.1 Existing Radar Classification Methods

Describe:

Threshold-based classification

Handcrafted Doppler features

Classical machine-learning models

1D CNN classification

2D spectrogram CNN classification

FPGA acceleration

3.2 Existing FPGA Deployment Approaches

Explain three main approaches.

Processor-only implementation

Simple but may not meet real-time latency requirements.

Vendor DPU implementation

Reduces development effort but limits architecture and supported operators.

Custom HLS accelerator

Provides control over precision, pipeline architecture, memory, interfaces, and parallelism, but requires more engineering effort.

3.3 Research Gap

The project addresses these gaps:

Window-level leakage is often not clearly discussed.

High accuracy may not reflect unseen-recording performance.

Many deployment projects stop after model compilation.

Many HLS CNN examples do not complete physical implementation.

Frame-buffered designs often exceed embedded FPGA memory.

3.4 Literature Review Table

Ref.	Work	Input	Model	Hardware	Split Method	Result	Gap

[1]	[Original paper]	[Insert]	[Insert]	[Insert]	[Insert]	[Insert]	[Insert]

[2]	Original repository	Radar features	[Insert]	[Insert]	[Insert]	[Insert]	2D KV260 extension

[3]	[Micro-Doppler study]	Spectrogram	CNN	GPU	[Insert]	[Insert]	FPGA deployment

[4]	[FPGA CNN study]	Image	CNN	FPGA	[Insert]	[Insert]	Radar-specific validation

Do not copy accuracy numbers until the exact dataset split and evaluation method have been confirmed.

CHAPTER 4 — PROPOSED SYSTEM AND PROJECT EVOLUTION

This chapter should be added early. It will make the report much stronger.

4.1 Final System Architecture

Radar IQ Recording

↓

Recording Eligibility Check

↓

256-Sample Windowing, Stride 32

↓

Top-3 Energy Range-Bin Averaging

↓

100×100 Spectrogram

↓

Depthwise-Separable CNN

↓

Four Output Logits

↓

Target Prediction

4.2 Hardware/Software Partition

Processing System

Reads radar data

Generates spectrogram

Allocates DDR buffers

Starts accelerator

Reads logits

Performs argmax

Displays or logs result

Programmable Logic

Executes fixed-point CNN

Performs convolution

Performs pooling

Performs global average pooling

Produces four logits

State clearly:

The current custom accelerator implements spectrogram-to-logits inference. Raw IQ-to-spectrogram preprocessing remains on the PS/CPU.

4.3 Project Evolution

Use this diagram:

Software CNN

↓

High Window-Level Accuracy

↓

Leakage Risk Identified

↓

Record-Wise Validation

↓

Grouped Five-Fold Validation

↓

Vitis AI DPU Deployment

↓

Frame-Buffered HLS Design

↓

BRAM/URAM Overflow

↓

Streaming HLS Redesign

↓

RTL Co-Simulation

↓

Vivado Timing Closure

↓

Physical Board Benchmark [Pending]

This evolution is one of the strongest parts of your project.

CHAPTER 5 — REQUIREMENTS, CONSTRAINTS, AND ASSUMPTIONS

5.1 Functional Requirements

ID	Requirement	Status

FR-01	Read radar recordings	Complete

FR-02	Generate 256-sample windows	Complete

FR-03	Generate 100×100 spectrograms	Complete

FR-04	Classify four classes	Complete

FR-05	Prevent recording overlap	Complete

FR-06	Export model weights	Complete

FR-07	Run fixed-point C simulation	Complete

FR-08	Generate RTL	Complete

FR-09	Verify RTL through co-simulation	Complete

FR-10	Integrate with Zynq PS	Complete

FR-11	Meet timing	Complete

FR-12	Run physical-board inference	Pending

FR-13	Benchmark performance	Pending

5.2 Important Assumptions

Include these explicitly:

The hardware input is a normalized 100×100 single-channel spectrogram.

Input normalization is identical to the training pipeline.

Class order is fixed:

0 Drone

1 Bird

2 Human

3 Corner Reflector

Batch size is one.

Weights are fixed at synthesis.

The accelerator receives 10,000 input values.

Four logits are returned.

PS software performs cache flush and cache invalidation.

RTL co-simulation latency is an estimate, not a physical-board measurement.

The dataset does not represent all possible ranges, weather, clutter, SNR, or target motion.

DPU and HLS results are only directly comparable when they use the same checkpoint and preprocessing.

5.3 Result Classification

Use labels throughout the report:

Label	Meaning

Verified	Confirmed by test or report

Estimated	Derived from synthesis or cycle count

Pending	Not yet performed

Assumed	Required operating condition

CHAPTER 6 — DATASET AND PREPROCESSING

6.1 Original Dataset

The original dataset is a NumPy object array with shape 130×6. Each source row contains an identifier, complex IQ data, metadata arrays, and label-related information.

6.2 Eligibility

For the grouped preprocessing run:

Total source rows: 130

Eligible rows: 59

Too-short rows: 71

Window length: 256

Stride: 32

Spectrogram size: 100×100

Range mode: topk_energy_mean

Top-k: 3

The grouped split produces train, validation, and test caches while maintaining zero recording-row overlap.

6.3 Why Top-3 Range-Bin Averaging Was Used

A single maximum-energy bin can be unstable when energy shifts between nearby range cells. Averaging the top three energetic bins provides a more stable representation.

6.4 Split Example

Split	Source Rows	Windows

Training	41	1115

Validation	9	226

Test	9	401

6.5 Data Imbalance

Explain that Drone is the majority class, while Bird, Human, and Corner Reflector contain fewer independent recordings.

Do not show only window counts. Show:

Window count per class

Recording count per class

Fold-wise test support

CHAPTER 7 — MACHINE LEARNING MODEL

7.1 Selected Model

The selected model is a depthwise-separable 2D CNN with approximately 24,612 trainable parameters.

7.2 Model Architecture

Stage	Operation	Channels

Input	100×100 spectrogram	1

Stem	3×3 convolution	16

Block 1	Depthwise + pointwise	32

Pool 1	2×2 max pooling	32

Block 2	Depthwise + pointwise	64

Pool 2	2×2 max pooling	64

Block 3	Depthwise + pointwise	96

Pool 3	2×2 max pooling	96

Block 4	Depthwise + pointwise	128

GAP	Global average pooling	128

Classifier	1×1 convolution	4

Insert the exact tensor shape after every stage from the actual PyTorch model.

7.3 Training Configuration

Parameter	Value

Initial learning rate	0.001

Weight decay	0.0001

Batch size	32

Maximum epochs	100

Early stopping patience	12

Scheduler patience	8

Selection metric	Validation macro F1

Augmentation	Training only

Imbalance method	Capped class weights

Maximum class weight	8.0

7.4 Why Macro F1 Was Used

Accuracy can be dominated by Drone samples. Macro F1 gives equal importance to every class and is therefore more suitable for checkpoint selection.

CHAPTER 8 — VALIDATION METHODOLOGY

8.1 Window-Level Result

Metric	Value

Accuracy	96.93%

Macro F1	92.54%

Weighted F1	96.98%

This should be described as an engineering result because overlapping windows may share source recordings between training and testing. The original project README already distinguishes this window-level result from the stricter record-wise result.

8.2 Strict Row-Wise Baseline

Metric	Value

Accuracy	43.74%

Macro F1	52.79%

Weighted F1	51.58%

Row overlap	0

8.3 Best Grouped Single Split

Metric	Value

Accuracy	94.26%

Macro F1	64.42%

Weighted F1	95.90%

Recording accuracy	88.89%

Row overlap	0

This is the best single split, not the final scientific result.

8.4 Grouped Five-Fold Result

Use this as the official result:

Metric	Mean ± Standard Deviation

Window accuracy	80.57% ± 2.98%

Window macro F1	66.45% ± 5.29%

Window weighted F1	83.24% ± 2.49%

Recording accuracy	86.67% ± 11.18%

Recording macro F1	74.04% ± 15.70%

Recording weighted F1	83.39% ± 12.67%

These are the strongest available recording-disjoint validation statistics.

8.5 Imbalance Strategy Comparison

Strategy	Accuracy	Macro F1	Weighted F1

Strong inverse weights	75.31%	54.81%	84.51%

Sampler only	88.53%	59.55%	92.58%

Capped weights	94.26%	64.42%	95.90%

The single-split comparison supports capped weights, but five-fold metrics remain the primary evidence.

CHAPTER 9 — VITIS AI DPU PATH

9.1 Flow

PyTorch

→ ONNX

→ Vitis AI Quantization

→ Quantized XModel

→ KV260 Compilation

→ VART Runtime

9.2 Linear Layer Issue

The original classifier used a Linear layer. Although it trained correctly, the compiler mapped the network path to CPU.

9.3 1×1 Convolution Workaround

The classifier was replaced by a 1×1 convolution, after which the main network mapped as:

USER → DPU → CPU

This confirms DPU compatibility.

9.4 Pending DPU Work

Physical VART run: [PENDING]

DPU inference latency: [PENDING]

DPU power: [PENDING]

DPU accuracy on board: [PENDING]

CHAPTER 10 — CUSTOM HLS ACCELERATOR

10.1 HLS Objective

The custom path was created so the project could claim:

CNN inference is implemented as a custom FPGA accelerator rather than relying only on the vendor DPU.

10.2 Top-Level Interfaces

Interface	Function

s_axi_control	Start, status, pointer registers

m_axi_gmem0	Read input spectrogram

m_axi_gmem1	Write logits

interrupt	Completion indication

ap_clk	Clock

ap_rst_n	Reset

10.3 Fixed-Point Arithmetic

Insert the exact final declarations:

data_t = [Insert exact ap_fixed type]

acc_t  = [Insert exact ap_fixed type]

State that saturation is used to avoid wrap-around corruption.

CHAPTER 11 — STREAMING REDESIGN

11.1 Initial Frame-Buffered Architecture

The original design stored complete intermediate tensors between CNN stages.

Examples:

Stem output: 16×100×100

PW1 output: 32×100×100

PW2 output: 64×50×50

This caused excessive BRAM and UltraRAM use.

11.2 Streaming Solution

The final design uses:

hls::stream

FIFO channels

Three-row line buffers for 3×3 convolution

Two-row buffering for pooling

Channel-vector tokens for pointwise convolution

DATAFLOW for overlapping execution

11.3 Resource Evolution

Architecture	DSP	BRAM	URAM	Fits K26

Initial forced-URAM HLS	~817	~777	~229	No

No-URAM frame-buffered HLS	~817	~1600	0	No

Final streaming implementation	681	25.5	0	Yes

11.4 RTL Co-Simulation

Top-level reported latency:

224,409 cycles

Do not directly call this the physical-board latency.

Use:

Estimated latency = 224,409 / implemented clock frequency

Leave spaces:

Implemented frequency: [________ MHz]

Estimated compute time: [________ ms]

Measured hardware time: [________ ms]

CHAPTER 12 — VIVADO INTEGRATION

12.1 Block Design

Describe:

Zynq UltraScale+ PS

↓

AXI SmartConnect

↓

Radar CNN HLS IP

↓

High-Performance DDR Port

12.2 Control Interface

The accelerator AXI-Lite control interface was mapped at:

0xA0000000

Confirm this address after final XSA export.

12.3 Memory Interfaces

gmem0 reads the input spectrogram.

gmem1 writes four logits.

Both access PS DDR through the enabled high-performance AXI port.

12.4 Clock and Reset

Describe the PL clock, Processor System Reset block, active-low accelerator reset, SmartConnect reset, and unused optional reset outputs.

12.5 Interrupt

State whether the accelerator interrupt is connected:

Interrupt connection status: [CONNECTED / PENDING]

CHAPTER 13 — VERIFICATION AND TESTING

13.1 Verification Chain

PyTorch

→ Host-Float C++

→ Fixed-Point C Simulation

→ HLS Synthesis

→ RTL Co-Simulation

→ Vivado Synthesis

→ Vivado Implementation

→ Board Test [Pending]

13.2 Verification Matrix

Test	Result

Dataset load	Pass

Spectrogram generation	Pass

Source-row overlap	Pass, zero overlap

PyTorch inference	Pass

ONNX validation	Pass

Host-float equivalence	Pass

Fixed-point C simulation	Pass

RTL co-simulation	Pass

Resource fit	Pass

Timing closure	Pass

Board inference	Pending

Full hardware accuracy	Pending

Performance measurement	Pending

CHAPTER 14 — RESULTS AND DISCUSSION

14.1 Final Hardware Utilization

Based on your post-implementation report:

Resource	Used	Available	Utilization

LUT	97,365	117,120	83.1%

FF	71,696	234,240	30.6%

DSP	681	1,248	54.6%

BRAM	25.5	144	17.7%

URAM	0	64	0%

14.2 Timing

Parameter	Result

WNS	+0.068 ns

TNS	0.000 ns

Failing endpoints	0

Timing constraints	Met

14.3 Interpretation

The design fits within all major resource limits. LUT utilization is high but acceptable for the present design. DSP and BRAM usage leave reasonable margin. The successful timing closure demonstrates that the streaming accelerator is physically implementable at the selected clock.

14.4 Main Engineering Achievement

The strongest hardware result is the change from:

Complete Feature Maps

→ Excessive BRAM/URAM

→ Design Cannot Fit

to:

Streaming + Line Buffers + DATAFLOW

→ 25.5 BRAM

→ 0 URAM

→ Timing-Closed Design

CHAPTER 15 — COMPARATIVE ANALYSIS

15.1 Recommended ML Comparison

Version	Split	Accuracy	Macro F1	Interpretation

Initial	Window	96.93%	92.54%	Optimistic

Strict baseline	Record-wise	43.74%	52.79%	Honest baseline

Improved	Grouped 5-fold	80.57% ± 2.98%	66.45% ± 5.29%	Official result

15.2 Recommended Hardware Comparison

Parameter	Frame-Buffered	Streaming

Full feature maps	Yes	No

DATAFLOW	No/limited	Yes

DSP	~817	681

BRAM	~777–1600	25.5

URAM	0–229	0

Fits KV260	No	Yes

Timing closure	Not applicable	Achieved

15.3 CPU/DPU/HLS Comparison

Leave empty until measured.

Platform	Model	Precision	Latency	FPS	Power	Accuracy

Arm CPU	[Model]	[Float]	[Pending]	[Pending]	[Pending]	[Pending]

Vitis AI DPU	[Model]	[INT8]	[Pending]	[Pending]	[Pending]	[Pending]

Custom HLS	[Model]	[Fixed point]	[Pending]	[Pending]	[Pending]	[Pending]

For a fair comparison, all three must use the same:

Checkpoint

Spectrogram

Normalization

Class order

Timing method

CHAPTER 16 — LIMITATIONS AND FUTURE WORK

16.1 Limitations

Dataset imbalance.

Small number of independent Bird and Human recordings.

Spectrogram preprocessing remains on CPU.

Physical-board results are pending.

No power benchmark.

No live radar stream.

High LUT usage limits additional PL features.

Compile-time weights.

The system is not certified for operational radar use.

16.2 Immediate Future Work

Program the KV260.

Create the host software.

Run one known spectrogram.

Compare all four logits against C simulation.

Run the complete held-out set.

Measure latency and throughput.

Compare CPU, DPU, and HLS.

Measure board power.

16.3 Advanced Improvements

Hardware STFT/spectrogram generation.

AXI DMA.

Ping-pong buffers.

INT8 or mixed precision.

Runtime-loadable weights.

Unknown-target rejection.

Confidence thresholding.

Live radar ADC interface.

Multi-frame recording-level voting.

Hardware cycle counters.

CHAPTER 17 — CONCLUSION

The project successfully progressed from radar IQ preprocessing and software CNN training to a custom, timing-closed FPGA implementation. The initial high window-level accuracy was treated cautiously after identifying the risk created by correlated overlapping windows. Record-wise and grouped five-fold validation were introduced to provide a stronger assessment of model generalization.

On the hardware side, the initial HLS implementation was functionally correct but exceeded device memory because full intermediate feature maps were stored. Replacing the frame-buffered architecture with a streaming DATAFLOW pipeline reduced on-chip memory consumption sufficiently to fit the K26 device. The final design passed RTL co-simulation, used 681 DSPs and 25.5 BRAMs, required no UltraRAM, and met timing with positive setup slack.

The remaining work is physical-board execution and measured benchmarking. Once completed, the project will provide a complete comparison between software CPU inference, Vitis AI DPU deployment, and a custom HLS CNN accelerator.

Better Ideas for the Final Report

Put “Project Evolution” Immediately After the Introduction

This shows that the project was not a simple successful run; it involved identifying and solving real problems.

Add an Evidence Traceability Matrix

Claim	Evidence	Status

Zero recording overlap	Split manifest	Verified

Model accuracy	Test metrics	Verified

HLS C correctness	C-sim log	Verified

RTL correctness	Co-sim report	Verified

Resource fit	Utilization report	Verified

Timing closure	Timing report	Verified

Physical acceleration	Board test	Pending

Speedup	Benchmark	Pending

Keep “Estimated” and “Measured” Separate

Do not call the 224,409-cycle co-simulation result a measured board latency. Report it as an RTL estimate until physical timing is collected.

Use the Five-Fold Result as the Main Accuracy Claim

The strongest defensible statement is:

Grouped five-fold recording-disjoint validation achieved 80.57% ± 2.98% window-level accuracy and 66.45% ± 5.29% macro F1, with zero source-recording overlap.

Do Not Hide Failed Designs

Include the failed frame-buffered design. It creates a much stronger engineering story:

Initial design failed resource constraints

→ bottleneck identified

→ streaming redesign

→ resource reduction

→ timing closure

Add These Figures

Dataset structure.

Window-generation process.

Leakage example.

Spectrograms for all four classes.

CNN architecture.

DPU flow.

Frame-buffered HLS architecture.

Streaming HLS architecture.

Resource comparison chart.

Vivado block design.

Timing-closure screenshot.

Board demonstration photograph [PENDING].

The original project documentation already contains the DPU artifact structure and distinguishes the optional HLS path from the DPU path; the final report should update that older status because the custom HLS flow has now progressed through implementation and timing closure.